| CAE LAB #2 | Combined Stress | 21900416 Gyeonheal An |
| --- | --- | --- |

I. PreLAB

- What is Generalized Hookes Law and present it in a formula.

When the material at a point is subjected to a state of triaxial stress, σx, σy, σz, then these stresses can be related to the normal strains ϵx, ϵy, ϵz by using the principle of superposition, Poisson’s ratio, ϵlat=-vϵlong and Hooke’s law as it applies in the uniaxial direction, ϵ=σ/E.

Figure  1. Bending stress in the Generalized Hooke's Law

ϵx=1Eσx-vσy+σz

ϵy=1Eσy-vσx+σz

ϵz=1Eσz-vσx+σy

Figure  2. Shear stress in the Generalized Hooke's Law

γxy=1Gτxy,      γyz=1Gτyz,     γxz=1Gτxz

- When a strain gauge rosette (a sensor with three strain gauges) is attached to the surface of an object to measure 2D strains (ϵx, ϵy, ϵz), it allows us to determine the 2D stress state of the surface (σx, σy, τxy) Please provide the mathematical expressions for this process using a 45-degree rosette gauge.

εx'=εxcos2θ+εysin2θ+γxysinθcosθ

for+45°,   εa=εx2+εy2+γxy2for 0°,                  εb=εxfor-45°,   εc=εx2+εy2-γxy2

∴εx=εb,            εy=εa+εc-εb,  γxy=εa-εc

σx=E(εx+vεy)(1-v2),          σy=E(εy+vεx)(1-v2),         τxy=Gγxy=Eγxy2(1+v)

- Investigate how to find the maximum shear stress, maximum tensile stress (principal stresses), and Von-Mises stress once you know the 2D stress state of the surface (σx, σy, τxy).

σ1,2=σx+σy2±σx-σy22+τxy2

τmax =σx-σy22+τxy2

σVM=σx2+σy2-σxσy+3τxy2

II. Main LAB

- Introduction

Figure  3. Composite Stress Testing Equipment

The composite stress testing equipment involves the use of a micrometer head to apply a vertical load to an L-shaped beam, with the aim of determining the stress at the strain gauge attachment point. The position of the micrometer head, which is mounted on LM guides, can be adjusted on the L-shaped beam. While this adjustment doesn't affect the bending stress, it does affect the torsional stress. Consequently, the composite stress varies depending on the micrometer's position.

Using NI-DAQ USB-6001 and LabVIEW coding, the voltage of the strain rosette is measured, and the strain state is determined by applying the strain rosette equation. From this information, the stress state is calculated, and through stress transformation, the principal stresses and maximum shear stress are determined.

- Experimental Results

- Checking the gain value

To verify the gain of the amplifier, press the calibration (CAL) button on each of the three strain gauge amplifiers in sequence and record the output voltage values in Table 2. For a gain of 1500, the value near 2.25V should be obtained.

|  | Gauge_a (+45°) | Gauge_b (0°) | Gauge_c (-45°) |
| --- | --- | --- | --- |
| eout-CAL [V} | 2.20 | 2.23 | 2.30 |

Table 1. Calibration Voltage of Three Gauges

- Elasticity Coefficient Estimation Experiment

To obtain the elastic modulus through regression analysis, record the displacement values for loads of 1, 2, and 3 kg, as well as the voltage values of the strain gauge b (0° gauge) in a table. Pay attention to the hysteresis of the displacement gauge.

| Load [kg] | Displacement [mm] | eout V for Gauge_b |
| --- | --- | --- |
| 1 | 2.67 | 0.34 |
| 2 | 5.31 | 0.69 |
| 3 | 8.40 | 1.08 |

Table 2. δ (displacement) and Output voltage of Gauge_b

- Stress Measurement Experiment

Record three experimental values of the strain rosette voltage when a 3 kg load is applied in a table.

| Count | eout for Gaugea [V] | eout for Gaugeb [V] | eout for Gaugec [V] |
| --- | --- | --- | --- |
| 1 | +0.82 | +0.95 | -0.25 |
| 2 | +0.84 | +0.93 | -0.32 |
| 3 | +0.81 | +0.93 | -0.19 |
| Average | +0.823 | +0.937 | -0.253 |

Table 3. Output, Average Voltage of Three Gauges

- Discussion

- Compare and analyze the elastic modulus obtained through two methods: the Deflection Method and the Strain Method.

- Strain Method

eout=∆R4R×E×Gamp;          R=120Ω,  E=5V,           Gamp=1500

ϵ=∆RR×1GF;      GF=2.13

σ=McI=m×9.81×150×340×6312

σ=Eϵ

| Load [kg] | eout V for Gauge_b | ∆R [Ω] | ϵ | σ [Mpa] | E [Gpa] |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.34 | 0.02176 | 8.513×10-5 | 6.13 | 72 |
| 2 | 0.69 | 0.04416 | 1.728×10-4 | 12.26 | 70.9 |
| 3 | 1.08 | 0.06912 | 2.704×10-4 | 18.39 | 68 |

Table 4. Elasticity obtained through the Strain Method

The elasticity value obtained above, when analyzed using linear regression in Matlab, results in the following.

Figure  4. Linear regression graph (Strain Method)

E = 74.335 [Gpa]

- Deflection Method

δ=PL33EI,          E=PL33δI=m×9.81×33533×δ×40×6312

| Load [kg] | Displacement [mm] | E |
| --- | --- | --- |
| 1 | 2.67 | 63.95 |
| 2 | 5.31 | 64.31 |
| 3 | 8.40 | 60.98 |

Table 5. Elasticity obtained by Deflection Method

The elasticity value obtained above, when analyzed using linear regression in Matlab, results in the following.

Figure  5. Linear regression graph (Deflection Method)

E = 66.083 [Gpa]

- Compare and analyze the experimental and theoretical values of bending stress (σx) and shear stress (τxy).

- First, we determined the change in resistance using the same method as in discussion 1 based on the output voltage obtained from the experiment. Then, we calculated the rate of change and used the generalized Hooke's law to obtain bending and shear stresses. The Young's modulus used in this calculation was the value obtained using the strain method in discussion 1 (74.335), and the Poisson's ratio (v) was assumed as 0.33 for AL-6061-T6.

|  | Gaugea | Gaugeb | Gaugec |
| --- | --- | --- | --- |
| eout | 0.823 | 0.937 | -0.253 |
| ∆R | 0.053 | 0.059 | -0.016 |
| ϵ | 2.0735E-4 | 2.308E-4 | -6.259E-5 |

Table 6. The resistance change and strain for the three strain gauges

εx=εb=2.308×10-4,   εy=εa+εc-εb=-8.604×10-5

σx=Eεx+vεy1-v2=16.88 Mpa

σy=Eεy+vεx1-v2=-0.824 [Mpa]

By Generalized Hooke’s Law, experimental value of shear stress,

γxy=εa-εc=2.6995×10-4

τxy=Gγxy=Eγxy2(1+v)=7.54 [Mpa]

- Theoretical value of bending stress,

σx=McI=3×9.81×150×340×6312=18.39 [Mpa]

Theoretical value of shear stress,

τmax=Tcbh2,   c=131-0.63hb

τmax=τxy=3×9.81×0.113×1-0.63×0.0060.04×0.04×0.0062=6.77 Mpa

The maximum shear stress acting on a square cross-section subjected to torsion occurs at the midpoint of the section. Therefore, in the experiment, strain gauges were attached to the center of the surface for measurement, and as a result, the calculated maximum value matches the theoretical value for the xy coordinates.

|  | σx [Mpa] | τxy [Mpa] |
| --- | --- | --- |
| Experimental Value | 16.88 | 7.54 |
| Theorical Value | 18.39 | 6.77 |

Table 7. Comparison of experimental and theoretical bending and shear stress

- Compare and analyze the experimental, theoretical, and simulation values for principal stress (σ1), maximum shear stress (τmax), and Von-Mises stress (σvm)

- In experimental,

σ1,2=σx+σy2±σx-σy22+τxy2=19.656, -3.59 [Mpa]

τmax=σx-σy22+τxy2=11.628 Mpa

σvm=σx2+σy2-σxσy+3τxy2=21.68 [Mpa]

- In theory,

σ1,2=σx+σy2±σx-σy22+τxy2=20.613, -2.22 Mpa

τmax=σx-σy22+τxy2=11.89 [Mpa]

σvm=σx2+3τxy2=21.81 [Mpa]

- In simulation,

To observe the point at 150mm, we positioned the probe at 130mm, taking into account the coordinate system's origin, and verified the results.

Figure  6. Maximum principal stress in ANSYS simulation

σ1=20.653  [Mpa]

Figure  7. Maximum shear stress in ANSYS simulation

τmax=11.428 [Mpa]

Figure  8. Von-mises stress in ANSYS simulation

σvm=21.839 Mpa

|  | σ1 | τmax | σvm |
| --- | --- | --- | --- |
| Experiment | 19.656 | 11.628 | 21.68 |
| Theory | 20.613 | 11.89 | 11.428 |
| Simulation | 20.653 | 11.428 | 21.839 |

- To ensure a safety factor of 3 and continue with this experiment next year, what is the maximum load that can be applied to the L-shaped beam?

SF=3= σyσvm_max

σvm_max=σx2+3τxy2=McI2 +3×Tcbh22

=m×9.81×0.335×0.0030.04×0.0063122+3×m×9.81×0.113×1-0.63×0.0060.04×0.04×0.00622

=1.875×1014×m2+1.528×1013×m2=2.0278×1014×m

m=280×1063×2.0278×1014=6.554 [kg]

- Please discuss any observations and lessons learned from the experiment, in addition to the points mentioned above.

- The area where stress is concentrated when a composite force is applied to the L-beam.

Figure  9. Von-Misses Stress

When torque was applied to the L-beam of a cantilever, it was observed that stress is concentrated closer to the center, indicating that this is due to the presence of stress in the y-direction when complex stresses occur. This was demonstrated through simulation.

Figure  10. Maximum Shear and Principal Stress

This applies similarly to maximum shear stress and maximum principal stress. Through this, it can be inferred that the y-direction stress had an influence when calculating the three types of stress based on experimental data in discussion 3.

- Distribution of shear stress

Figure  11. Shear stress distribution

When torque is applied to a square cross-section, it was confirmed through simulation that the central portion of the surface of the square cross-section has the highest shear stress, as indicated by the formula in discussion 2.

- Appendix

- Geometry Design (DesignModeler)

Figure  12. Geometry of L-Beam

- Material (AL-6061-T6)

Table 8. Material properties (AL-6061-T6)

- Experimental Condition

| Used Material | AL-6061-T6 |
| --- | --- |
| Mesh | 3mm |
| Force | -29.43N (3kg) |
| Solution | Total Deformation |
|  | Equivalent Stress |
|  | Directional Deformation |
|  | Maximum Shear Stress |
|  | Maximum Principal Stress |

Table 9. Conditions

- Boundary Conditions & Load Conditions

Table 10. BC & LC

- Linear regression function using Matlab

Figure  13. Linear regression Code

- References

- R.C. Hibbeller, “Mechanics of Materials”, 8th Ed, Pearson, 2012

- Professor CS.Lee’s CAE textbook (2023) from HGU Mechanical Control Engineering